ARCHITECTURE DESCRIPTION DOCUMENT

T-150/TRV-150 Heavy-Lift UAS Simulation

AFSIM Project 005

Version 1.0
19 December 2025

UNCLASSIFIED // FOR OFFICIAL USE ONLY

# Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2025-12-19 | Project Team | Initial release |
| 0.9 | 2025-12-01 | Project Team | Draft for review |

# Table of Contents

(Auto-generate in Word: References > Table of Contents)

1. Introduction
   1.1 Purpose
   1.2 Threat System Description
   1.3 Simulation Framework
   1.4 Design Standards
2. Model Architecture Description
   2.1 Architecture Overview
   2.2 Plugin Component Descriptions
   2.3 Mathematical Models
   2.4 Inter-Processor Communication
   2.5 Design Decisions
3. Sources and References
4. Acronyms and Abbreviations

# 1. Introduction

## 1.1 Purpose

This Architecture Description Document (ADD) describes the design, implementation, and mathematical foundations of the T-150/TRV-150 Heavy-Lift UAS simulation developed using AFSIM (Advanced Framework for Simulation, Integration, and Modeling). This document provides the technical details necessary for verification and validation (V&V) of the simulation model, including the algorithms, design choices, and inter-component interfaces.

## 1.2 Threat System Description

The T-150/TRV-150 is a heavy-lift unmanned aerial system manufactured by Malloy Aeronautics and marketed as the SURVICE TRV-150. It is a coaxial octocopter configuration with 8 rotors mounted on 4 arms, designed for cargo delivery and logistics missions.

### 1.2.1 Platform Specifications

| Parameter | Value | Source |
| --- | --- | --- |
| Configuration | Coaxial Octocopter (8 rotors, 4 arms) | Technical Documentation |
| Motors | 8x T-Motor U15II KV100 | FAA Registry |
| Total Thrust | 240 kg | AAIB Report |
| MTOW | 123 kg (271 lbs) | AAIB Report |
| Max Payload | 68 kg (150 lbs) | Manufacturer |
| Empty Weight | ~55 kg (estimated) | Derived |
| Cruise Speed | 30 m/s (108 km/h) | Manufacturer |
| Max Airspeed | 40+ m/s | AAIB (demonstrated) |
| Max Rate of Climb | 8 m/s | Derived |
| Max Rate of Descent | 4 m/s (VRS limit) | Aerodynamic Analysis |
| Battery | 13S Li-Ion, 46.8V nominal | US Patent |
| Battery Capacity | ~7 kWh (148.75 Ah) | US Patent |
| Flight Time | 36 minutes (nominal) | Manufacturer |

## 1.3 Simulation Framework

The simulation is built on AFSIM version 2.2505, utilizing the WSF_FULL_ROTORCRAFT_MOVER for flight dynamics and custom C++ processor plugins for subsystem modeling. The framework provides table-driven performance characteristics with mode-based operation (DEFAULT, HOVER) and realistic rotorcraft kinematics.

| Component | Description |
| --- | --- |
| AFSIM 2.2505 | Core simulation framework with entity-based modeling |
| WSF_FULL_ROTORCRAFT_MOVER | Table-driven rotorcraft dynamics with PID control |
| WsfProcessor Plugins | Custom C++ plugins for battery, propulsion, VRS, slung load |
| AuxData Interface | Inter-processor communication via key-value store |

## 1.4 Design Standards

The simulation adheres to the following design principles:

Processor Independence: Each processor functions independently with fallback values

Loose Coupling: Processors communicate via AuxData, not direct references

Script Accessibility: All key values accessible from scenario scripts

Real-Time Execution: Plugins operate at 100ms update intervals (10 Hz)

Stable Numerics: Quadratic solutions and semi-implicit Euler integration for stability

# 2. Model Architecture Description

## 2.1 Architecture Overview

The T-150 UAS simulation extends AFSIM with four custom processor plugins that model the engineering subsystems of the aircraft. These plugins communicate through the platform's AuxData container, providing loose coupling and script accessibility.

### 2.1.1 System Block Diagram

+------------------------------------------------------------------+
|                       AFSIM Platform                              |
|                                                                   |
|  +----------------+   +----------------+   +------------------+   |
|  | Propulsion     |   | Battery        |   | VRS              |   |
|  | Processor      |   | Processor      |   | Processor        |   |
|  |                |   |                |   |                  |   |
|  | - Motor states |-->| - SOC tracking |   | - Descent rate   |   |
|  | - Thermal model|   | - Voltage sag  |   | - VRS margin     |   |
|  | - Ground effect|   | - Energy usage |   | - State machine  |   |
|  | - Payload mass |   |                |   |                  |   |
|  +-------+--------+   +--------+-------+   +--------+---------+   |
|          |                     |                    |             |
|          v                     v                    v             |
|  +----------------------------------------------------------+    |
|  |                  Platform AuxData                         |    |
|  |  PROP_TOTAL_POWER, BATTERY_SOC, VRS_MARGIN, SLUNG_*       |    |
|  +----------------------------------------------------------+    |
|          |                                                        |
|          v                                                        |
|  +----------------------------------------------------------+    |
|  |              Platform Mover (Rotorcraft)                  |    |
|  |          GetVelocityNED(), GetAltitude()                  |    |
|  +----------------------------------------------------------+    |
|                                                                   |
|  +------------------+                                             |
|  | Slung Load       |                                             |
|  | Processor        |                                             |
|  | - Pendulum dyn.  |                                             |
|  | - Cable tension  |                                             |
|  +------------------+                                             |
+------------------------------------------------------------------+

### 2.1.2 Plugin Summary

| Plugin | Class | Purpose |
| --- | --- | --- |
| t150_battery | BatteryProcessor | Battery SOC, voltage sag, energy tracking |
| t150_propulsion | PropulsionProcessor | Motor power, thermal, RPM, thrust, ground effect |
| t150_vrs | VRSProcessor | Vortex Ring State detection and warning |
| t150_slung_load | SlungLoadProcessor | Spherical pendulum dynamics for cable loads |

## 2.2 Plugin Component Descriptions

### 2.2.1 Battery Processor (T150_BATTERY_PROCESSOR)

The Battery Processor models a 13S Li-Ion battery pack with realistic voltage sag behavior under load. It tracks state of charge (SOC), computes terminal voltage using a polynomial OCV curve, and provides warnings for low battery and voltage sag conditions.

#### Configuration Parameters

| Parameter | Default | Units | Description |
| --- | --- | --- | --- |
| battery_capacity_wh | 7000.0 | Wh | Total battery capacity |
| initial_soc | 1.0 | 0-1 | Initial state of charge |
| hover_power_w | 18000.0 | W | Fallback hover power |
| low_battery_warning | 0.20 | 0-1 | SOC threshold for warning |
| nominal_voltage | 46.8 | V | 13S x 3.6V nominal |
| full_charge_voltage | 54.6 | V | 13S x 4.2V full |
| cutoff_voltage | 39.0 | V | 13S x 3.0V cutoff |
| internal_resistance | 0.025 | Ohms | Pack internal resistance |
| voltage_sag_warning | 42.0 | V | Voltage sag warning threshold |
| use_dynamic_power | false | bool | Read power from propulsion |

### 2.2.2 Propulsion Processor (T150_PROPULSION_PROCESSOR)

The Propulsion Processor models the 8-motor coaxial propulsion system. It includes motor thermal dynamics, ground effect augmentation, payload mass tracking, and provides total power consumption data to the battery processor.

#### Configuration Parameters

| Parameter | Default | Units | Description |
| --- | --- | --- | --- |
| motor_count | 8 | - | Number of motors |
| kv_rating | 100.0 | RPM/V | Motor KV rating |
| max_power_per_motor | 3500.0 | W | Maximum power per motor |
| propeller_diameter | 0.76 | m | 30" propeller diameter |
| coaxial_efficiency | 0.85 | 0-1 | Coaxial interference factor |
| motor_thermal_limit | 120.0 | C | Motor thermal limit |
| motor_thermal_mass | 1500.0 | J/K | Motor thermal mass |
| motor_cooling_rate | 20.0 | W/K | Forced convection cooling |
| motor_time_constant | 0.15 | s | RPM response time constant |
| ground_effect_enabled | true | bool | Enable ground effect |
| empty_mass | 68.0 | kg | Empty aircraft mass |
| payload_mass | 0.0 | kg | Current payload mass |
| max_payload_mass | 55.0 | kg | Maximum payload capacity |

### 2.2.3 VRS Processor (T150_VRS_PROCESSOR)

The VRS Processor detects Vortex Ring State conditions based on descent rate and forward speed. It implements a 5-state machine (CLEAR, CAUTION, ONSET, DEVELOPED, RECOVERY) and computes lift and control degradation factors.

#### Configuration Parameters

| Parameter | Default | Units | Description |
| --- | --- | --- | --- |
| vrs_descent_threshold | 4.0 | m/s | Descent rate for VRS onset |
| vrs_speed_threshold | 5.0 | m/s | Forward speed to exit VRS |
| caution_margin | 0.8 | 0-1 | Margin for caution state |
| induced_velocity | 8.0 | m/s | Hover induced velocity |
| coaxial_factor | 0.85 | 0-1 | Coaxial VRS resistance factor |
| vrs_development_time | 2.0 | s | Time to develop full VRS |
| vrs_recovery_time | 3.0 | s | Time to recover from VRS |
| max_lift_degradation | 0.6 | 0-1 | Maximum lift loss in VRS |

### 2.2.4 Slung Load Processor (T150_SLUNG_LOAD_PROCESSOR)

The Slung Load Processor models cable-suspended payload dynamics using a spherical pendulum approximation. It computes swing angles, cable tension, and load forces on the aircraft, with resonance detection and overload protection.

#### Configuration Parameters

| Parameter | Default | Units | Description |
| --- | --- | --- | --- |
| load_mass | 0.0 | kg | Suspended load mass |
| cable_length | 5.0 | m | Cable length |
| cable_damping | 0.1 | 1/s | Damping coefficient |
| attach_point_z | 0.5 | m | Attach point below CG |
| max_cable_tension | 5000.0 | N | Maximum cable tension |
| resonance_threshold | 20.0 | deg | Swing angle for warning |
| update_interval | 0.02 | s | 50 Hz dynamics update |

## 2.3 Mathematical Models

### 2.3.1 Battery Voltage Sag Model

The battery models realistic Li-Ion voltage behavior under load using open-circuit voltage (OCV), internal resistance, and a stable quadratic solution for current draw.

#### Open-Circuit Voltage (OCV)

The OCV varies with SOC following a polynomial approximation of the Li-Ion discharge curve:

V_cell = 3.0 + 1.2*SOC - 0.5*SOC2 + 0.5*SOC3

This produces per-cell voltages of ~4.2V at 100% SOC, ~3.6V at 50% SOC, and ~3.0V at 0% SOC. The pack voltage is computed by multiplying the cell voltage by the cell count (13 for a 13S pack).

#### Quadratic Current Solution

For a constant power load P with internal resistance R and open-circuit voltage OCV, the terminal voltage and current are related by:

P = V_terminal * I and V_terminal = OCV - I*R

Substituting and solving the quadratic equation:

I = (OCV - sqrt(OCV2 - 4*R*P)) / (2*R)

This quadratic solution is stable because it directly solves for current without the feedback instability that can occur with iterative I = P/V calculations.

#### SOC-Dependent Internal Resistance

Internal resistance follows a U-shaped curve with minimum around 60% SOC:

SOC > 60%: R_factor = 1.0 + 0.3*(SOC - 0.6)^2 / 0.16 (peaks ~1.3x at 100%)

SOC 20-60%: R_factor = 1.0 (minimum resistance)

SOC < 20%: R_factor = 1.0 + 2.0*(0.2 - SOC)^2 / 0.04 (peaks ~3x at 0%)

### 2.3.2 Motor Thermal Model

Each motor accumulates heat from electrical losses and dissipates via forced convection:

dT/dt = (Q_in - Q_out) / C_thermal

Where:

Q_in = P_electrical * (1 - efficiency) [Heat from losses]

Q_out = k_cooling * (T_motor - T_ambient) [Convective cooling]

C_thermal = 1500 J/K [Motor thermal mass]

k_cooling = 20 W/K [Cooling rate from prop wash]

### 2.3.3 Motor Response Dynamics

Motor RPM follows a first-order lag model for actuator dynamics:

alpha = dt / (tau + dt)

RPM_new = RPM_old + alpha * (RPM_target - RPM_old)

With tau = 150ms time constant for large motors with 30" props.

### 2.3.4 Ground Effect Model

Ground effect thrust augmentation uses the Cheeseman-Bennett approximation:

T_IGE / T_OGE = 1 / (1 - (R / 4h)2)

Where:

T_IGE = Thrust in ground effect

T_OGE = Thrust out of ground effect

R = Rotor radius (0.38m for 30" props)

h = Rotor height above ground

Ground effect is active below 1 rotor diameter AGL (0.76m) with maximum boost clamped to 30% (1.3x thrust multiplier).

### 2.3.5 Vortex Ring State Detection

VRS occurs when high descent rate causes the rotor to descend through its own wake. The VRS margin combines descent rate and forward speed factors:

descent_margin = 1.0 - (descent_rate / threshold)

speed_margin = min(1.0, forward_speed / speed_threshold)

Forward speed above 5 m/s provides sufficient fresh air inflow to prevent VRS. Coaxial rotors have inherent VRS resistance (0.85 factor applied to thresholds).

#### VRS State Machine

| State | Condition | Effect |
| --- | --- | --- |
| CLEAR | VRS margin >= 1.0 | No degradation |
| CAUTION | margin < 0.8 | 5% lift loss |
| ONSET | margin < 0.5 | 15% lift loss developing |
| DEVELOPED | margin < 0.2 for 2s | 60% lift loss, 50% control loss |
| RECOVERY | margin increasing | Degradation decreasing over 3s |

### 2.3.6 Spherical Pendulum Dynamics

The slung load is modeled as a spherical pendulum with two rotational degrees of freedom:

theta_x'' = -(g/L)*sin(theta_x) - (a_x/L)*cos(theta_x) - c*theta_x'

theta_y'' = -(g/L)*sin(theta_y) - (a_y/L)*cos(theta_y) - c*theta_y'

Where:

theta_x, theta_y = Swing angles in X-Z and Y-Z planes (rad)

g = Gravitational acceleration (9.81 m/s^2)

L = Cable length (m)

a_x, a_y = Aircraft accelerations in X and Y (m/s^2)

c = Damping coefficient (1/s)

#### Cable Tension

T = m * (g*cos(theta) - a_z + L*omega^2)

Including centripetal acceleration from swing angular velocity.

#### Natural Frequency

f = (1/2*pi) * sqrt(g/L)

For a 5m cable: f = 0.22 Hz (4.5 second period).

#### Numerical Integration

Semi-implicit Euler integration is used for stability:

1. Update velocity: theta_dot += theta_dot_dot * dt

2. Update position: theta += theta_dot * dt

3. Clamp angles to +/- 70 degrees to prevent numerical blowup

## 2.4 Inter-Processor Communication

Processors communicate through the platform's AuxData container, a key-value store that provides loose coupling and script accessibility.

### Published AuxData Keys

#### Propulsion Processor Outputs

| Key | Type | Description |
| --- | --- | --- |
| PROP_TOTAL_POWER | double | Total electrical power draw (W) |
| PROP_TOTAL_THRUST | double | Combined motor thrust (N) |
| PROP_MAX_TEMP | double | Hottest motor temperature (C) |
| PROP_AVG_TEMP | double | Average motor temperature (C) |
| PROP_MOTOR_COUNT | int | Operational motor count |
| PROP_SYSTEM_EFF | double | Overall system efficiency |
| PROP_GROUND_EFFECT | double | Ground effect factor (1.0 = none) |
| PROP_ALTITUDE_AGL | double | Altitude above ground (m) |
| PROP_TOTAL_MASS | double | Empty + payload mass (kg) |
| PROP_THRUST_MARGIN | double | Available thrust margin (0-1) |
| PROP_CG_OFFSET_X/Y/Z | double | Combined CG offset (m) |
| PROP_OVERLOADED | bool | Payload exceeds capacity |

#### Battery Processor Outputs

| Key | Type | Description |
| --- | --- | --- |
| BATTERY_SOC | double | State of charge (0-1) |
| BATTERY_POWER_W | double | Current power consumption (W) |
| BATTERY_ENERGY_WH | double | Remaining energy (Wh) |
| BATTERY_VOLTAGE | double | Terminal voltage under load (V) |
| BATTERY_OCV | double | Open-circuit voltage (V) |
| BATTERY_VOLTAGE_SAG | double | Voltage drop from IR (V) |
| BATTERY_CURRENT_A | double | Current draw (A) |
| BATTERY_LOW | bool | Low battery warning flag |

#### VRS Processor Outputs

| Key | Type | Description |
| --- | --- | --- |
| VRS_STATE | int | Current VRS state enum (0-4) |
| VRS_MARGIN | double | Safety margin (0=VRS, 1=safe) |
| VRS_DESCENT_RATE | double | Current descent rate (m/s) |
| VRS_LIFT_FACTOR | double | Lift degradation factor (0-1) |
| VRS_CONTROL_FACTOR | double | Control authority factor (0-1) |
| VRS_CAUTION | bool | Caution warning flag |
| VRS_CRITICAL | bool | Critical VRS warning flag |

## 2.5 Design Decisions

### 2.5.1 Why WSF_FULL_ROTORCRAFT_MOVER

The WSF_FULL_ROTORCRAFT_MOVER was selected over WSF_BASIC_ROTORCRAFT_MOVER for:

Table-driven performance varying with altitude and airspeed

Mode-based operation (DEFAULT cruise, HOVER for station-keeping)

More realistic rotorcraft dynamics with PID control

Acceleration tables for realistic deceleration profiles

### 2.5.2 Why Quadratic Voltage Solution

The quadratic solution for battery current was chosen over iterative I=P/V because:

Iterative solutions can diverge when voltage drops cause current spikes

Quadratic directly solves without feedback loops

Handles edge cases (discriminant < 0 means power exceeds capability)

Computationally efficient (one sqrt vs multiple iterations)

### 2.5.3 Why 100ms Update Interval

The 100ms (10 Hz) update interval balances accuracy and performance:

Battery and propulsion dynamics are slow (thermal time constants > 1s)

VRS detection needs reasonable response time but not real-time control

Matches AFSIM typical processor update rates

Slung load uses 20ms (50 Hz) for pendulum dynamics stability

### 2.5.4 Why AuxData for Communication

The AuxData interface was chosen for inter-processor communication because:

Loose coupling: Processors work independently without direct references

Script access: Scenario scripts can read/write values

Debugging: All values visible in AFSIM data inspection tools

Fallback support: Processors can use defaults if peer unavailable

# 3. Sources and References

[1] AAIB Accident Investigation: Air Accidents Investigation Branch report on TRV-150 incident at South Scarle, Nottinghamshire, UK, 2023

[2] US Patent US20240262499A1: Battery topology and power management for heavy-lift UAS

[3] FAA Aircraft Registry: TRV-150 registration and configuration data

[4] T-Motor U15II Specifications: Motor performance data: KV100, 3.5kW max power

[5] AFSIM Documentation: AFSIM 2.2505 framework documentation, WSF_FULL_ROTORCRAFT_MOVER

[6] Cheeseman & Bennett (1955): Ground effect model: "The Effect of the Ground on a Helicopter Rotor"

[7] Johnson, W. (1980): Helicopter Theory, Princeton University Press - VRS conditions

[8] Prouty, R. (1986): Helicopter Performance, Stability, and Control - rotorcraft dynamics

# 4. Acronyms and Abbreviations

| Acronym | Definition |
| --- | --- |
| ADD | Architecture Description Document |
| AFSIM | Advanced Framework for Simulation, Integration, and Modeling |
| AGL | Above Ground Level |
| CG | Center of Gravity |
| ESC | Electronic Speed Controller |
| IGE | In Ground Effect |
| IR | Internal Resistance |
| KV | Motor velocity constant (RPM per Volt) |
| MTOW | Maximum Takeoff Weight |
| NED | North-East-Down (coordinate frame) |
| OCV | Open-Circuit Voltage |
| OGE | Out of Ground Effect |
| PID | Proportional-Integral-Derivative |
| RPM | Revolutions Per Minute |
| SOC | State of Charge |
| UAS | Unmanned Aerial System |
| V&V | Verification and Validation |
| VRS | Vortex Ring State |
| WSF | Warfighter Simulation Framework |
| Wh | Watt-hour |