USER AND ANALYST MANUAL

T-150/TRV-150 Heavy-Lift UAS Simulation

AFSIM Project 005

Version 1.0
19 December 2025

UNCLASSIFIED // FOR OFFICIAL USE ONLY

# Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2025-12-19 | Project Team | Initial release |
| 0.9 | 2025-12-01 | Project Team | Draft for review |

# Table of Contents

(Auto-generate in Word: References > Table of Contents)

1. Introduction
2. Threat System Description
3. Model Description
4. Threat Model Components
5. Basic Setup and General Guide
6. Analytic Outputs
7. Model Fidelity Log
8. Sources and References
9. Acronyms and Abbreviations

# 1. Introduction

## 1.1 Purpose

This User and Analyst Manual (UAM) provides comprehensive guidance for operating, modifying, and extending the T-150/TRV-150 Heavy-Lift UAS simulation. The document is intended for simulation analysts, developers, and researchers who need to:

Run simulations with various mission profiles and configurations

Build and install custom processor plugins

Modify platform parameters and flight characteristics

Integrate the simulation with other AFSIM scenarios

Extend the model with new capabilities

## 1.2 Scope

This manual covers the complete T-150 UAS simulation including:

Platform definition with WSF_FULL_ROTORCRAFT_MOVER flight dynamics

Battery processor with Li-Ion voltage sag model

Propulsion processor with thermal, ground effect, and payload models

VRS (Vortex Ring State) detection processor

Slung load dynamics processor

Scenario construction and mission planning

Plugin development and build process

## 1.3 Prerequisites

Before using this simulation, ensure you have:

AFSIM 2.2505 or later installed at /opt/afsim2.2505/

C++ compiler (GCC 9+ or compatible) for plugin development

CMake 3.16+ for building plugins

Basic familiarity with AFSIM scenario scripting

Linux environment (WSL2 or native Linux)

# 2. Threat System Description

## 2.1 Platform Overview

The T-150/TRV-150 is a heavy-lift unmanned aerial system designed for logistics and cargo delivery missions. Key characteristics include:

| Characteristic | Value |
| --- | --- |
| Configuration | Coaxial Octocopter (8 rotors, 4 arms) |
| MTOW | 123 kg (271 lbs) |
| Max Payload | 68 kg (150 lbs) internal, 55 kg slung |
| Cruise Speed | 30 m/s (108 km/h) |
| Max Range | 70 km |
| Flight Time | 36 minutes (nominal) |
| Service Ceiling | 3000 m |
| Battery | 13S Li-Ion, 7 kWh capacity |
| Motors | 8x T-Motor U15II KV100 |
| Propellers | 30" (0.76m) diameter |
| Hover Power | ~18 kW |

## 2.2 Concept of Operations (CONOPS)

The T-150 is designed for autonomous cargo delivery missions with the following typical mission profile:

Takeoff: Vertical climb to cruise altitude (typically 100-500m AGL)

Cruise: Point-to-point or waypoint-following flight at 30 m/s

Station-keeping: Hover capability for precision operations

Payload delivery: Land or slung load release

Return: Autonomous return to launch point

## 2.3 Operational Limitations

CRITICAL FLIGHT ENVELOPE LIMITS VRS onset: Descent rate > 4 m/s with forward speed < 5 m/s Maximum bank angle: 45 deg (operational), 60 deg (demonstrated) Battery cutoff voltage: 39V (3.0V/cell) - mission abort required

| Parameter | Operational | Do Not Exceed |
| --- | --- | --- |
| Max Airspeed | 30 m/s | 40 m/s |
| Max Climb Rate | 5 m/s | 8 m/s |
| Max Descent Rate | 3 m/s | 4 m/s (VRS limit) |
| Max Bank Angle | 35 deg | 60 deg |
| Motor Temperature | 100 C | 120 C |
| Battery Voltage | > 42V | > 39V |

# 3. Model Description

## 3.1 Model Overview

The T-150 simulation consists of a platform definition using AFSIM's WSF_FULL_ROTORCRAFT_MOVER for flight dynamics, enhanced by four custom C++ processor plugins that model engineering subsystems.

### 3.1.1 System Block Diagram

+-------------------------------------------------------------------+
|                    T-150 UAS Simulation Model                      |
+-------------------------------------------------------------------+
|                                                                    |
|  +---------------------+    +-----------------------------+        |
|  | Platform Definition |    | Custom Processors           |        |
|  | (t150_platform)     |    | (C++ Plugins)               |        |
|  +---------------------+    +-----------------------------+        |
|  |                     |    |                             |        |
|  | - T150_MOVER        |    | - T150_BATTERY_PROCESSOR    |        |
|  | - T150_RADAR_SIG    |    | - T150_PROPULSION_PROCESSOR |        |
|  |                     |    | - T150_VRS_PROCESSOR        |        |
|  +---------------------+    | - T150_SLUNG_LOAD_PROCESSOR |        |
|           |                 +-----------------------------+        |
|           v                              |                         |
|  +---------------------+                 |                         |
|  | WSF_FULL_ROTORCRAFT |<----------------+                         |
|  | _MOVER              |   (AuxData Communication)                 |
|  +---------------------+                                           |
|           |                                                        |
|           v                                                        |
|  +---------------------+                                           |
|  | Scenario Definition |                                           |
|  | (Route, Execute,    |                                           |
|  |  Event Recording)   |                                           |
|  +---------------------+                                           |
|                                                                    |
+-------------------------------------------------------------------+

## 3.2 Intended Use

This simulation is intended for:

Mission planning and route optimization studies

Battery endurance and energy consumption analysis

Flight envelope exploration and limitation studies

Payload delivery mission analysis

Slung load operations assessment

VRS encounter scenario development

Sensor/payload integration studies

## 3.3 Key Performance Indicators (KPIs)

| KPI | AuxData Key | Description |
| --- | --- | --- |
| Battery SOC | BATTERY_SOC | State of charge (0-1) |
| Terminal Voltage | BATTERY_VOLTAGE | Under-load voltage (V) |
| Power Consumption | PROP_TOTAL_POWER | Total power draw (W) |
| Motor Temperature | PROP_MAX_TEMP | Hottest motor temp (C) |
| VRS Margin | VRS_MARGIN | VRS safety margin (0-1) |
| Thrust Margin | PROP_THRUST_MARGIN | Available thrust (0-1) |
| Ground Effect | PROP_GROUND_EFFECT | GE thrust factor |
| Cable Tension | SLUNG_CABLE_TENSION | Slung load tension (N) |

# 4. Threat Model Components

## 4.1 Platform Definition Files

### 4.1.1 t150_mover.afsim

Defines the WSF_FULL_ROTORCRAFT_MOVER with T-150 flight characteristics:

mover_type T150_MOVER WSF_FULL_ROTORCRAFT_MOVER
   rotorcraft_kinematics
      performance_limits
         available_airspeed   30 m/s     // Cruise speed
         maximum_rate_of_ascent 5 m/s    // Operational climb
         maximum_rate_of_descent 3 m/s   // Safe descent (VRS margin)
      end_performance_limits

      global_limits
         maximum_airspeed     40 m/s     // Never exceed
         maximum_rate_of_ascent 8 m/s    // Physical limit
         maximum_rate_of_descent 4 m/s   // VRS onset threshold
      end_global_limits

      acceleration_table               // Deceleration profile
         speed    0 m/s   deceleration 0.5 m/s^2
         speed   10 m/s   deceleration 2.5 m/s^2
         speed   30 m/s   deceleration 3.0 m/s^2
      end_acceleration_table
   end_rotorcraft_kinematics
end_mover_type

### 4.1.2 t150_platform.afsim

Defines the platform type combining mover and processors:

platform_type T150_UAS WSF_PLATFORM
   mover T150_MOVER

   processor BATTERY_MON T150_BATTERY_PROCESSOR
      battery_capacity_wh    7000.0
      initial_soc            1.0
      nominal_voltage        46.8
      internal_resistance    0.025
      use_dynamic_power      true
   end_processor

   processor PROP_MON T150_PROPULSION_PROCESSOR
      motor_count            8
      max_power_per_motor    3500
      empty_mass             68.0
      payload_mass           40.0
   end_processor

   processor VRS_MON T150_VRS_PROCESSOR
      vrs_descent_threshold  4.0
      vrs_speed_threshold    5.0
   end_processor
end_platform_type

## 4.2 Plugin Source Code

The simulation includes four C++ processor plugins:

| Plugin Directory | Source Files |
| --- | --- |
| plugins/t150_battery/ | BatteryProcessor.cpp, BatteryProcessor.hpp |
| plugins/t150_propulsion/ | PropulsionProcessor.cpp, PropulsionProcessor.hpp |
| plugins/t150_vrs/ | VRSProcessor.cpp, VRSProcessor.hpp |
| plugins/t150_slung_load/ | SlungLoadProcessor.cpp, SlungLoadProcessor.hpp |

# 5. Basic Setup and General Guide

## 5.1 Project Directory Structure

project005_t150_uas/
├── README.md                    # Project overview
├── platforms/
│   ├── t150_mover.afsim         # Rotorcraft mover definition
│   └── t150_platform.afsim      # Platform type definition
├── scenarios/
│   └── t150_flight_demo.afsim   # Example mission scenario
├── plugins/
│   ├── CMakeLists.txt           # Top-level CMake
│   ├── build.sh                 # Build script
│   ├── install_plugins.sh       # Install to AFSIM
│   ├── t150_battery/            # Battery plugin source
│   ├── t150_propulsion/         # Propulsion plugin source
│   ├── t150_vrs/                # VRS plugin source
│   └── t150_slung_load/         # Slung load plugin source
├── output/                      # Simulation output files
├── tools/                       # Python analysis scripts
├── docs/                        # Documentation
└── VandV_docs/                  # V&V documentation

## 5.2 Building Plugins

IMPORTANT: Plugin Installation Plugins MUST be installed to the AFSIM system directory (/opt/afsim2.2505/lib/wsf_plugins/) to take effect. The local build directory is only used for compilation.

Step 1: Navigate to plugins directory and build

cd /home/shepard/afsim_projects/project005_t150_uas/plugins
./build.sh

# Expected output:
# -- Configuring done
# -- Generating done
# -- Build files written to: .../plugins/build
# [100%] Built target libt150_battery.so
# [100%] Built target libt150_propulsion.so
# [100%] Built target libt150_vrs.so
# [100%] Built target libt150_slung_load.so

Step 2: Install plugins (requires sudo)

./install_plugins.sh

# This copies:
#   build/lib/libt150_battery.so -> /opt/afsim2.2505/lib/wsf_plugins/
#   build/lib/libt150_propulsion.so -> /opt/afsim2.2505/lib/wsf_plugins/
#   build/lib/libt150_vrs.so -> /opt/afsim2.2505/lib/wsf_plugins/
#   build/lib/libt150_slung_load.so -> /opt/afsim2.2505/lib/wsf_plugins/

## 5.3 Running Simulations

### 5.3.1 Command Line Execution

# Navigate to project directory
cd /home/shepard/afsim_projects/project005_t150_uas

# Run the simulation
/opt/afsim2.2505/bin/mission scenarios/t150_flight_demo.afsim

# Output files will be created in output/ directory

### 5.3.2 Visualization with Mystic

# After running simulation, view in Mystic
DISPLAY=:0 /opt/afsim2.2505/bin/mystic output/t150_flight_demo.aer

# For headless systems (WSL2), use X server like VcXsrv

## 5.4 Creating Custom Scenarios

A minimal scenario requires:

// my_scenario.afsim
include ../platforms/t150_platform.afsim

simulation
   time 0 sec
   time_step 0.1 sec
   end_time 600 sec
end_simulation

platform t150_drone T150_UAS
   route
      // Starting position
      position 52.9 deg -1.1 deg  altitude 0 m  speed 0 m/s

      // Climb to altitude
      position 52.9 deg -1.1 deg  altitude 100 m  speed 0 m/s

      // Fly to destination
      position 52.91 deg -1.1 deg  altitude 100 m  speed 30 m/s

      // Hover at destination
      position 52.91 deg -1.1 deg  altitude 100 m  speed 0 m/s

      // Return to start
      position 52.9 deg -1.1 deg  altitude 100 m  speed 30 m/s

      end_route loiter
   end_route
end_platform

// Enable event recording
event_pipe
   file output/my_scenario.aer
   platform_track_messages on
end_event_pipe

## 5.5 Modifying Platform Parameters

To customize the platform for different configurations, modify the processor parameters in your scenario or platform definition file:

// Example: Heavy payload configuration
platform_type T150_HEAVY_LOAD T150_UAS
   processor PROP_MON T150_PROPULSION_PROCESSOR
      motor_count            8
      max_power_per_motor    3500
      empty_mass             68.0
      payload_mass           55.0        // Maximum payload
      max_payload_mass       55.0
   end_processor

   processor BATTERY_MON T150_BATTERY_PROCESSOR
      battery_capacity_wh    7000.0
      hover_power_w          22000.0     // Higher for heavy load
      use_dynamic_power      true
   end_processor
end_platform_type

// Example: Slung load configuration
platform_type T150_SLUNG T150_UAS
   processor SLUNG_MON T150_SLUNG_LOAD_PROCESSOR
      load_mass              40.0        // 40 kg slung load
      cable_length           10.0        // 10 meter cable
      cable_damping          0.15
      max_cable_tension      6000.0      // Rated cable strength
   end_processor
end_platform_type

## 5.6 Plugin Development Guide

### 5.6.1 Plugin Structure

Each plugin follows the AFSIM WsfProcessor pattern:

// MyProcessor.hpp
class MyProcessor : public WsfProcessor
{
public:
    MyProcessor(const WsfScenario& aScenario);
    static WsfStringId GetMyProcessorClassId();

    // Required overrides
    WsfProcessor* Clone() const override;
    bool ProcessInput(UtInput& aInput) override;
    bool Initialize(double aSimTime) override;
    void Update(double aSimTime) override;
    const char* GetScriptClassName() const override;

private:
    // Configuration parameters
    double mMyParameter;

    // Runtime state
    double mLastUpdateTime;
};

### 5.6.2 Registering the Plugin

// In MyProcessor.cpp
#include "WsfProcessorClassInfo.hpp"
#include "MyProcessor.hpp"

// Register plugin with AFSIM
WSF_PROCESSOR_REGISTER(MyProcessor, "MY_PROCESSOR",
    "Description of my processor");

### 5.6.3 CMakeLists.txt Template

cmake_minimum_required(VERSION 3.16)
project(my_plugin)

# Find AFSIM
set(AFSIM_ROOT "/opt/afsim2.2505")
include(${AFSIM_ROOT}/share/cmake/afsim.cmake)

# Add plugin library
afsim_add_plugin(my_plugin
    source/MyProcessor.cpp
    source/MyProcessor.hpp
)

## 5.7 Accessing AuxData from Scripts

Scenario scripts can read AuxData values published by processors:

// In scenario execute block
execute at_time 60 sec
   // Read battery state
   double soc = t150_drone.BATTERY_SOC
   double voltage = t150_drone.BATTERY_VOLTAGE

   // Check for warnings
   if (soc < 0.2)
      write_log "Low battery warning: " soc*100 "%"
   end_if

   // Read propulsion state
   double power = t150_drone.PROP_TOTAL_POWER
   double max_temp = t150_drone.PROP_MAX_TEMP

   // Read VRS state
   double vrs_margin = t150_drone.VRS_MARGIN
   if (vrs_margin < 0.5)
      write_log "VRS warning: margin = " vrs_margin
   end_if
end_execute

# 6. Analytic Outputs

## 6.1 Output Files

| File Type | Extension | Contents |
| --- | --- | --- |
| Event Record | .aer | Platform tracks, events, for Mystic visualization |
| Log File | .log | Console output, warnings, processor messages |
| Track Data | .csv | Position/velocity data (if configured) |
| Script Output | .txt | Custom script output via write_file |

## 6.2 Available Telemetry Data

The following data can be logged or monitored during simulation:

### 6.2.1 Platform State

Position: Latitude, Longitude, Altitude (MSL/AGL)

Velocity: Ground speed, Vertical speed, Heading

Attitude: Roll, Pitch, Yaw (if available from mover)

Route status: Current waypoint, distance to waypoint

### 6.2.2 Subsystem State (via AuxData)

| Category | Available Parameters |
| --- | --- |
| Battery | SOC, Voltage, OCV, Current, Power, Voltage Sag |
|  | Low Battery Flag, Voltage Low Flag |
| Propulsion | Total Power, Total Thrust, Motor Count |
|  | Max/Avg Temperature, System Efficiency |
|  | Ground Effect Factor, Altitude AGL |
| Payload | Total Mass, Payload Mass, Thrust Margin |
|  | CG Offset X/Y/Z, Overloaded Flag |
| VRS | VRS State (0-4), VRS Margin |
|  | Descent Rate, Forward Speed |
|  | Lift Factor, Control Factor |
|  | Caution/Onset/Critical Flags |
| Slung Load | Load State, Load Mass, Cable Length |
|  | Cable Tension, Swing X/Y/Magnitude |
|  | Force X/Y/Z, Position X/Y/Z |
|  | Natural Frequency, Resonance Flag |

## 6.3 Analysis Scripts

Python analysis scripts are provided in the tools/ directory for post-processing:

plot_battery.py - Battery SOC, voltage, current over time

plot_thermal.py - Motor temperature profiles

plot_trajectory.py - 3D flight path visualization

analyze_energy.py - Energy consumption by flight phase

# 7. Model Fidelity Log

## 7.1 Fidelity Assessment

| Component | Fidelity | Basis | Limitations |
| --- | --- | --- | --- |
| Flight Dynamics | Medium | Table-driven kinematic | Not 6-DOF physics |
| Battery Model | Medium-High | Li-Ion polynomial OCV | No temperature effects |
| Motor Thermal | Medium | First-order thermal | Simplified cooling model |
| Ground Effect | Medium | Cheeseman-Bennett | Flat terrain assumed |
| VRS Detection | Medium | Empirical thresholds | Simplified state machine |
| Slung Load | Medium | Spherical pendulum | Small angle approx at high swing |
| Payload/CG | Low-Medium | Static mass offset | No dynamic CG effects |

## 7.2 Known Limitations

No Wind Model: Atmospheric wind effects not modeled; affects energy and trajectory

Flat Terrain: Ground effect uses MSL altitude; no terrain database

Static Payload: Payload mass fixed at scenario start; no dynamic loading

No Sensor Noise: All measurements are perfect; no sensor error models

Simplified Acoustics: No acoustic signature model

Single Platform: No swarm or multi-vehicle coordination

## 7.3 Enhancement Roadmap

| Phase | Status | Description |
| --- | --- | --- |
| Phase 5 | Pending | Wind effects integration |
| Phase 6 | Pending | BEMT rotor aerodynamics |
| Phase 7 | Pending | Sensor noise injection |
| Phase 8 | Pending | Multi-vehicle operations |

# 8. Sources and References

[1] AAIB Accident Investigation: Air Accidents Investigation Branch report on TRV-150 incident at South Scarle, Nottinghamshire, UK, 2023

[2] US Patent US20240262499A1: Battery topology and power management for heavy-lift UAS

[3] FAA Aircraft Registry: TRV-150 registration and configuration data

[4] T-Motor U15II Specifications: Motor performance data: KV100, 3.5kW max power

[5] AFSIM Documentation: AFSIM 2.2505 framework documentation

[6] WSF_FULL_ROTORCRAFT_MOVER: AFSIM rotorcraft mover documentation

[7] Cheeseman & Bennett (1955): Ground effect approximation

[8] Helicopter VRS Theory: Vortex Ring State conditions and recovery

# 9. Acronyms and Abbreviations

| Acronym | Definition |
| --- | --- |
| AFSIM | Advanced Framework for Simulation, Integration, and Modeling |
| AGL | Above Ground Level |
| BEMT | Blade Element Momentum Theory |
| CG | Center of Gravity |
| CONOPS | Concept of Operations |
| DOF | Degrees of Freedom |
| ESC | Electronic Speed Controller |
| GE | Ground Effect |
| IGE | In Ground Effect |
| KPI | Key Performance Indicator |
| KV | Motor velocity constant (RPM per Volt) |
| MSL | Mean Sea Level |
| MTOW | Maximum Takeoff Weight |
| NED | North-East-Down (coordinate frame) |
| OCV | Open-Circuit Voltage |
| OGE | Out of Ground Effect |
| PID | Proportional-Integral-Derivative |
| RPM | Revolutions Per Minute |
| SOC | State of Charge |
| UAM | User and Analyst Manual |
| UAS | Unmanned Aerial System |
| V&V | Verification and Validation |
| VRS | Vortex Ring State |
| WSF | Warfighter Simulation Framework |
| Wh | Watt-hour |